Annexure A: Scope of Work – Employee Transfer Boat Service

1. Purpose & Service Description

1.1 The Vendor shall provide a dedicated passenger transfer boat service on a four-shift basis for the movement of employees between Shiyalbet Jetty (Mainland) and Shiyalbet Boat Landing Jetty (Island) and return, ensuring safe, timely, and efficient transportation.
1.2 The service shall operate in accordance with GPPL directives, Navigation advisories, tide and weather conditions, and applicable regulations of the Gujarat maritime Board, District administration and of other relevant authorities.
1.3 The indicative timetable, crew roster, and maximum passenger capacity per voyage shall be specified and may be varied by the Company with reasonable notice.

### 2. Vessel Technical Requirements

2.1 Vessel Type & Configuration
The vessel shall be a motorized passenger launch or equivalent, suitable for operations in the unsheltered waters of Port Pipavav and for jetty-to-jetty transfers. It must include:

- Enclosed or semi-enclosed passenger seating area with canopy/roof, non-slip decks, and adequate handrails.

- Safe freeboard with guard rails to prevent falls; boarding ladder or gangway fitted with handrails.

- Robust fendering system for jetty approaches and berthing to safeguard passengers and hull integrity. Fendering must be designed to withstand monsoon sea and swell conditions, as inadequate protection poses a risk of structural damage.

- Based on the above requirements, the vessel shall be constructed using suitable materials, with the hull made of steel, aluminium, or other suitable High impact resistance composite fibre that can sustain the monsoonal sea conditions.

- Bilge pumping arrangement comprising one automatic electric pump and one manual pump.

- Navigation lights compliant with COLREGs, audible signalling device/horn, and searchlight for low-light operations.

- Reliable propulsion: twin-engine configuration preferred; alternatively, a single engine with a documented redundancy plan and standby vessel availability.

- Service speed parameters: minimum 2 knots and maximum 12 knots, subject to verification during acceptance trials.

- Applications that do not meet the boat specifications will be opted out and will not be considered for further processing

2.2 Navigation & Communication

- VHF radiotelephone (one fixed and one handheld unit) maintaining continuous watch on channels designated by Port Control.

- GPS, magnetic compass, and AIS Class B (recommended) for enhanced situational awareness within port limits.

- Mobile communication backup and emergency contact list prominently displayed onboard.

2.3 Safety & Fire Protection (LSA/FFA)

- Lifejackets for at least 120% of maximum passenger capacity (adult and child sizes), properly stowed and marked.

- Minimum two lifebuoys, with at least one buoyant line and one light; lifebuoys to be readily accessible.

- Minimum two Class ABC fire extinguishers, including coverage for engine space; fire bucket with lanyard.

- First-aid kit stocked for minor injuries, stretcher or rescue board, and thermal blankets.

- Reflective markings and battery/solar-powered lighting for 360° visibility during night operations.

- Hull paint maintenance as per GPPL HSSE specifications.

- All LSA and FFA to be maintained under routine inspection as per GMB Vessel Survey Certification requirements.

- Maintaining monthly inspection records of the boat, including all safety and life-saving equipment.

- Vendor shall ensure crew is trained in using safety and life saving equipment. (GPPL facilitate training)

- Vendor to provide medical fitness certificates for all crew. (Later on GPPL facilitate during annual health checkup)

2.4 Pollution Prevention & Spill Control

- Strict prohibition on discharge of oil, waste, or effluents. Vessel to maintain drip trays, absorbent pads, and a mini-spill kit (SOPEP-lite).

- Waste collection and disposal through authorized facilities in compliance with environmental regulations.

2.5 Documentation & Certification

- Valid vessel registration, fitness/seaworthiness certificates, and crew competency documents issued by competent local authority; applicable inland/passenger vessel permits.

- Insurance certificates as per Clause 10.

- Latest survey/inspection reports and maintenance logs to be maintained onboard.

### 3. Crew Requirements

3.1 Minimum Manning per Shift
Crew strength shall comply with the Certificate of Survey or Minimum Manning Certificate issued by the competent authority. At a minimum, the vessel must be manned by a Master/Coxswain and an Engine Driver. Combining roles is permissible only if allowed under local regulations and supported by a documented risk assessment ensuring safe operations.

3.2 Competence & Approvals
All crew members shall possess valid certifications and approvals as mandated by Gujarat Maritime Board (GMB) or other competent authorities and must be medically fit for duty.

3.3 Training
Crew shall demonstrate proficiency in basic safety procedures, crowd management, first aid, firefighting, man-overboard recovery, and passenger safety briefings.

3.4 Fatigue Management
Duty hours and rest periods shall be managed to prevent fatigue. Strict prohibition on alcohol or drugs applies. Personal Protective Equipment (PPE)—including lifejackets, gloves, safety shoes, and helmets—must be worn as required.

3.5 Familiarization
Crew must be fully conversant with local port safety requirements, reporting protocols, route-specific hazards, and emergency response procedures.

### 4. Operations & Safety Management

4.1 Pre-Departure

- Conduct comprehensive pre-departure checks (engines, steering, bilge pumps, VHF, lights, LSA/FFA).

- Record passenger manifest; control boarding; manage seating; secure baggage.

- Deliver mandatory passenger safety briefing covering lifejacket use, seating discipline, prohibition on standing on gunwales, and emergency actions.

4.2 Routing & Speed
Operate strictly along designated routes and adhere to speed limits prescribed by the Port/Harbour Master. Maintain minimum wake near jetties and congested traffic areas.

4.3 Weather/Tide Criteria & Suspension
Suspend or restrict operations when wind, visibility, wave height, or tidal/current conditions exceed safe limits for the vessel type and loading. Thresholds shall align with the Zone of Operations defined in the Certificate of Survey and be reviewed seasonally. Decisions must be coordinated with Port Control, with timely stakeholder notifications and contingency plans (e.g., schedule adjustments, backup vessel deployment).

4.4 Communications
Maintain continuous watch on designated VHF channels. Report departures and arrivals to Port Control and ensure uninterrupted communication with Pipavav Port Control.

4.5 Night/Low-Light Operations
Deploy navigation lights, searchlights, and reflective tapes. Ensure adequate deck lighting for safe boarding and disembarkation. Implement enhanced lookout protocols.

4.6 Security & Conduct
Comply with port security directives. Prohibit unauthorized persons or cargo. Inspect baggage when required by security instructions.

### 5. Consumables, Fuel & Maintenance

5.1 Supply
Vendor shall provide diesel, lubricants, tools, tackles, and consumables at own cost to ensure uninterrupted service.

5.2 Preventive Maintenance
Implement scheduled maintenance programs. Record all defects and corrective actions.

5.3 Breakdown Response
Report faults immediately. Rectify within agreed timelines. Deploy a backup vessel if downtime exceeds SLA thresholds.

5.4 Cleanliness & Housekeeping
Maintain clean decks and seating areas. Prevent oil leaks. Dispose of waste responsibly.

### 6. Emergency Preparedness & Drills

6.1 Emergency Response Plan
Maintain a documented plan covering medical emergencies, fire, man-overboard, collision/allision, and grounding scenarios.

6.2 Drills
Conduct monthly crew drills and quarterly passenger drills/simulations. Maintain records of all exercises.

6.3 MOB Recovery
Stage rescue equipment and practice retrieval techniques. Coordinate promptly with Port Control and Company emergency contacts.

6.4 Medevac
Establish procedures for rapid return to mainland and handover to medical services.

### 7. Environmental Compliance

7.1 Zero Discharge Policy
Ensure immediate containment and reporting of accidental spills.

7.2 Compliance
Adhere to applicable environmental guidelines. Maintain spill response materials and conduct awareness toolbox talks.

### 8. Documentation & Reporting

8.1 Records
Maintain onboard and submit upon request: vessel and crew certificates, daily logs, passenger manifests, departure/arrival times, weather/tide notes, maintenance records, drill records, and incident/near-miss reports.

8.2 Data Privacy
Handle passenger data in accordance with Company policy. Retain only for statutory or operational requirements.

### 9. Performance Standards (SLAs & KPIs)

- Availability: ≥ 98% per month (excluding force majeure).

- On-Time Performance: ≥ 95% departures within ±10 minutes of schedule.

- Safety: Zero lost-time injuries; zero reportable pollution incidents.

- Reporting: Weekly SLA dashboard and monthly performance review with the Company.

### 10. Insurance

Vendor shall maintain at own cost:

- Hull & Machinery insurance for the vessel.

- Third-Party/Public Liability (including passenger liability) to limits acceptable to the Company.

- Workmen’s Compensation/Employer’s Liability for crew.
Certificates and endorsements must be submitted prior to mobilization and kept current.

### 11. Compliance & Audits

Ensure full compliance with Indian maritime laws, GMB requirements, inland/passenger vessel regulations, COLREGs, and environmental rules. The Company reserves the right to inspect/audit vessel, crew competence, records, and safety arrangements at any time. Vendor shall promptly address any findings.

### 12. Variations, Contingencies & Change Control

Accommodate changes to shift timings, capacity, or routes as directed by the Company within reasonable operational limits. Maintain contingency plans for adverse weather/tidal conditions or jetty unavailability.

### 13. Delay, Non-Performance & Penalties

In case of delay or failure, the Company may engage alternate service providers at Vendor’s risk and cost, including recovery of consequential expenses. Liquidated Damages (LDs) may apply for SLA breaches as per Commercial Annexure. Repeated non-compliance may result in termination for cause.

### 14. Mobilization, Acceptance & Handover

Prior to commencement, complete document verification, acceptance trials (route run, berthing, emergency drills), and crew familiarization. Rectify any deficiencies before operational go-live.

### 15. Health, Safety & Culture

Safety is paramount. Conduct toolbox talks before each shift. Perform Risk Assessment for operations and berthing. Encourage near miss reporting culture. Provide bilingual safety signage and briefings (English + Gujarati/Hindi) for passenger clarity.